Standard Operating Procedure (SOP)

| 1. | Contact Information |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Procedure Title: |  | ARC FLASH Event |  |  |  |  |
| Division: |  | Bus Maintenance |  |  |  |  |
| Department/Function: |  | [Specify] |  |  | SOP number: | [Specify] |
| Prepared by: |  | CALSTART | Date: | 1/6/2023 | Date issued: | [Specify] |
| Reviewed by: |  | [Specify] | Date: | [Specify] | Revision date: | [Specify] |
| Approved by: |  | [Specify] | Date: | [Specify] | Review date: | [Specify] |

| 2. | Purpose |
| --- | --- |
| Overview of ARC FLASH Event: an event that results from a connection through air to ground or another voltage phase in an electrical system. |  |

| 3. | Scope |
| --- | --- |
| Cover First Person/Responder on Scene from vehicle ARC FLASH Event. |  |

| 4. | Responsibility |
| --- | --- |
| First on scene, Supervisor. Identified in the Step-By-Step Operating Procedure. |  |

| 5. | Definitions |
| --- | --- |
| AED – Automatic External Defibrillator |  |

| 6. | Prerequisites (Training Requirements) |
| --- | --- |
| None. |  |

| 7. | Step-By-Step Operating Procedure |
| --- | --- |
| Responsible Party; Action Step. **** DO NOT TOUCH PERSON STILL IN CONTACT WITH SOURCE OF ELECTRICAL SHOCK *****. First on Scene; - If person is still in contact with electrical source, use a dry, non-conducting object, such as a wooden broom, chair, or hot stick to eliminate person’s contact with electrical source.. - If safe to do so, turn off power source by switching off the Battery Disconnect switch to OFF position (IMG 1.0).. - Turn the High Voltage Disconnect Switch to OFF Position (IMG 1.0).. - Check to see if person is breathing.. - If they are breathing, - Contact 911 first always. - If they are not breathing. - Designate a person to call 911. - Begin CPR and utilize an AED (Manual Defibrillator) and continue until Emergency Services arrives.. - If victim is on fire, flames can be smothered or doused. DO NOT REMOVE CLOTHES FROM VICTIM! Smother example: Cover with a blanket or jacket. Douse example: Use water to extinguish fire.. - Report Incident to Supervisor once Emergency Services has arrived and taken over if a Supervisor has not already been alerted.. |

| 7. | Step-By-Step Operating Procedure (continued) |
| --- | --- |
| Responsible Party; Action Step. **** DO NOT TOUCH PERSON STILL IN CONTACT WITH SOURCE OF ELECTRICAL SHOCK *****. Supervisor; - Ensure person is no longer in contact with electrical source of shock. If still in contact, use a dry, non-conducting object, such as a wooden broom, chair, or hot stick to eliminate person’s contact with electrical source. DO NOT ASSUME THIS HAS BEEN DONE!. - Verify with First Person on Scene the Battery Disconnect switch has been switched to the OFF position (see IMG 1.0).. - Verify with First Person on Scene that 911 have been called and follow 911 operator instructions.. - If person is showing signs of shock, such as vomiting or becoming faint or very pale, elevate the legs slightly, unless this causes too much pain.. - If possible, cover burns with sterile gauze. DO NOT USE ADHESIVE BANDAGES THAT MAY STICK TO THE BURN.. - Keep the person warm using available materials. (Example: blankets, jackets, coats). - Await rescue worker / EMT instruction.. |

| 8. | Equipment & Materials |
| --- | --- |
| TBD from Manufacturer IMG 1.0 |  |

| 9. | Health & Safety |
| --- | --- |
| None. |  |

| 10. | Waste Disposal |
| --- | --- |
| None. |  |

| 11. | Quality Assurance & Quality Control |
| --- | --- |
| None. |  |