Standard Operating Procedure (SOP)

| 1. | Contact Information |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Procedure Title: |  | Fire Event - Operator |  |  |  |  |
| Division: |  | Bus Maintenance |  |  |  |  |
| Department/Function: |  | [Specify] |  |  | SOP number: | [Specify] |
| Prepared by: |  | CALSTART | Date: | 1/6/2023 | Date issued: | [Specify] |
| Reviewed by: |  | [Specify] | Date: | [Specify] | Revision date: | [Specify] |
| Approved by: |  | [Specify] | Date: | [Specify] | Review date: | [Specify] |

| 2. | Purpose |
| --- | --- |
| To describe the procedural response to an emergency fire event on a battery electric bus. |  |

| 3. | Scope |
| --- | --- |
| Operator procedures with passengers on board. |  |

| 4. | Responsibility |
| --- | --- |
| Operator. Identified in the Step-By-Step Operating Procedure. |  |

| 5. | Definitions |
| --- | --- |
| None. |  |

| 6. | Prerequisites (Training Requirements) |
| --- | --- |
| None. |  |

| 7. | Step-By-Step Operating Procedure |
| --- | --- |
| Responsible Party; Action Step. Operator; - Move the vehicle to a safe location.. - Apply Parking Brake.. - Place shift to Neutral.. - Open ALL passenger doors.. - Shutdown the vehicle by setting the Master RUN switch to the OFF position.. - Direct all passengers to a safe area away from the vehicle and away from traffic.. - Alert Transit Authority with the following information: - Passenger status and count - Location - Situational status of the vehicle - Next steps that will be taken. Only proceed to the following steps if it is safe to do so. Approach the rear, curbside area of the vehicle.. Operator; - Approach rear curbside area of the vehicle and open the Battery Disconnect access door.. - Shut down the 12/24 VDC electrical power to the vehicle by setting the Battery Disconnect switch to the OFF position (see IMG 1.0).. - Turn the High Voltage Disconnect Switch to OFF Position (see IMG 1.0).. - Return to passenger safety area and wait for First Responders and Operational Supervisors.. |

| 8. | Equipment & Materials |
| --- | --- |
| To be provided by manufacturer IMG 1.0 |  |

| 9. | Health & Safety |
| --- | --- |
| None. |  |

| 10. | Waste Disposal |
| --- | --- |
| None. |  |

| 11. | Quality Assurance & Quality Control |
| --- | --- |
| None. |  |